Final tests

Шифр и наименование компетенции:

ОПК-11: готовностью к применению медицинских изделий, предусмотренных порядками оказания медицинской помощи пациентам со стоматологическими заболеваниями

Раздел дисциплины: Несъемное протезирование

Вид оценочного средства: Тестовые задания:

| Оценочное средство | Критерии оценки | Этап контроля |
| --- | --- | --- |
| 1. Materials for temporary fixation + repin - unifas + water dentin - JRC + Temp-Bond | 1, 3, 5 | ТК-РК, ПА |
| 2. Materials for permanent fixation - water dentin - dentin paste + visfat-cement + ketak molar + adhesive | 3,4,5 | ТК-РК, ПА |
| 3. Representatives of zinc phosphate cements: + unifas - JRC - repin + visfat | 1,4 | ТК-РК, ПА |
| 4. Liquid of glass ionomer cement is: - water solution of orthophosphoric acid - distilled water + water solution of polyacrylic acid | 3 | ТК-РК, ПА |
| 5. Glass ionomer cement powder contain: - zinc oxide, magnesium oxide, silicon oxide + aluminosilicate + fluorosilicate | 2,3 | ТК-РК, ПА |
| 6. Polycarboxylate cement fluid is: - water solution of orthophosphoric acid - distilled water + water solution of polyacrylic acid | 3 | ТК-РК, ПА |
| 7. Polycarboxylate cement powder contain: + modified zinc oxide - alumosilicate - fluorosilicate | 1 | ТК-РК, ПА |
| 8. Phosphate cement fluid is: + water solution of orthophosphoric acid - distilled water - polyacrylic acid | 1 | ТК-РК, ПА |
| 9. Phosphate cement powder contain: + zinc oxide, magnesium oxide, silicon oxide - alumosilicate - fluorosilicate | 1 | ТК-РК, ПА |
| 10. In the manufacture of pin stump tabs you must use wax composition: - modevax + lavax - byugelny wax | 2 | ТК-РК, ПА |
| 11. With the direct method of manufacturing the stump pin tab, the modeling is done by: + doctor - technician | 1 | ТК-РК, ПА |
| 12. What materials are used for the manufacture of the stump pin tab: + CSC + gold alloy - ftoraks - sildonton | 1,2 | ТК-РК, ПА |
| 13. Sterilization in the dry heat oven continues + at 180o 1 hour - at 180o 0.5 hours - at 160 about 2.5 hours - at 160 o 1 hour | 1 | ТК-РК, ПА |
| 14. What is the thickness of the metal stamped crown - 0.8-1.0 - 0.3-0.35 + 0.25-0.3 - 1.2-1.5 | 3 | ТК-РК, ПА |
| 15. What impression materials belong to silicones - gypsum + sielast -stomalgin - acrodent - Wanstein mass + zeta plus | 2,5 | ТК-РК, ПА |
| 16. What is the thickness of the porcelain crown - 0.3-0.35 - 0.25-0.3 - 0.8-1.0 + 1.5-2.0 | 4 | ТК-РК, ПА |
| 17. What impression materials are alginate? - gypsum - sielast, thiodent + stomalgin - orthocorre + alligat | 3,5 | ТК-РК, ПА |
| 18. Handpieces are sterilized by: + autoclave + boiling in liquid paraffin - boiling in distilled water - in a dry closet | 1,2 | ТК-РК, ПА |
| 19. A two-layer impression is obtained using the masses: - alginate - solid crystalline + silicone - thermoplastic - hydrocolloid | 3 | ТК-РК, ПА |

Раздел дисциплины: Съемное протезирование

Вид оценочного средства: Тестовые задания:

| Оценочное средство | Критерии оценки | Этап контроля |
| --- | --- | --- |
| 1. Occluders reproduce a. only vertical movements of the lower jaw b. sagittal and lateral movements of the mandible c. sagittal, lateral and vertical movements of the mandible d. only sagittal movements of the lower jaw e. only lateral movements of the lower jaw | 1 | ТК-РК, ПА |
| 2. Articulators reproduce a. sagittal, lateral and vertical movements of the mandible b. only vertical movements of the lower jaw c. only lateral movements of the lower jaw d. sagittal and vertical movements of the mandible e. sagittal and lateral movements of the mandible | 1 | ТК-РК, ПА |
| 3. Graphic method of registration of the chewing movements of the lower jaw a. masticography b. myotonography c. odontopodonogram d. rheoparodontography e. electromyography | 1 | ТК-РК, ПА |
| 4. The most biocompatible materials are a. ceramic masses b. metal alloys c. polymers d. hydrocolloids e. Composites | 1 | ТК-РК, ПА |
| 5. In the position of the central occlusion, the articular head of the mandible is a. at the base of the slope of the articular tubercle b. in the middle of the glenoid fossa c. at the top of the real knoll d. city behind the articular tubercle e. before articular tubercle | 1 | ТК-РК, ПА |

*- указывается, при каком уровне контроля применяется оценочное средство (текущий контроль, в том числе рубежный контроль (ТК, ТК-РК), промежуточная аттестация (ПА), государственная итоговая аттестация (ГИА).

ПК-5: готовность к сбору и анализу жалоб пациента, данных его анамнеза, результатов осмотра, лабораторных, инструментальных, патолого-анатомических и иных исследований в целях распознавания состояния или установления факта наличия или отсутствия стоматологического заболевания

Раздел дисциплины: Несъемное протезирование

Вид оценочного средства: Тестовые задания:

| Оценочное средство | Критерии оценки | Этап применения* |
| --- | --- | --- |
| 1. IROPZ in Milikevich 0.2-0.3. What design choose? + filling - tab - crown - pin tooth | 1 | ТК-РК, ПА |
| 2. IROPZ according to Milikevich 0.3–0.6 which structure to choose? - filling + inlay - crown - pin tooth | 2 | ТК-РК, ПА |
| 3. IROPZ in Milikevich 0.5-0.8. What design choose - seal - tab + crown - pin tooth | 3 | ТК-РК, ПА |
| 4. IROPZ in Milikevich more than 0.8. What treatment method choose - seal - tab - crown + pin construction | 4 | ТК-РК, ПА |
| 5. What tools are used for initial observation? + probe + mirror - putty knife - ironing machine | 1,2 | ТК-РК, ПА |
| 6. The physiological types of bite include: + biprognathic - deep - prognathic - cross - opened | 1 | ТК-РК, ПА |
| 7. The method of subjective examination of a patient in the clinic of orthopedic dentistry includes: - Inspection - Palpation + discussion - X-ray examination - Study of diagnostic models | 3 | ТК-РК, ПА |
| 8. Anatomical neck of the tooth corresponds to: + enamel transition in root cement - boundary above and subgingival part of the tooth - the equator of the tooth - the bottom of the periodontal groove - the bottom of the pathological periodontal caramna | 1 | ТК-РК, ПА |
| 9. What class of Black are cavities on the contact surfaces of molars and premolars? -1 +2 -3 -4 -5 | 2 | ТК-РК, ПА |
| 10. What class according to Black are the cavities on the contact surfaces of the front teeth with preservation of the cutting edge and angles -2 -1 +3 -5 -4 | 3 | ТК-РК, ПА |

*- указывается, при каком уровне контроля применяется оценочное средство (текущий контроль, в том числе рубежный контроль (ТК, ТК-РК), промежуточная аттестация (ПА), государственная итоговая аттестация (ГИА).

Раздел дисциплины: Съемное протезирование

Вид оценочного средства: Тестовые задания:

| Оценочное средство | Критерии оценки | Этап применения* |
| --- | --- | --- |
| 1. When planning the manufacture of a fixed structure, an insufficient number of abutment teeth can lead to a. functional overload of periodontium of abutment teeth b. increased antagonist abrasion c. multiple caries d. enamel fluorosis e. cleavage of orthopedic materials | 1 | ТК-РК, ПА |
| 2. In the long-span bridges, the corrosive process a. intensifies b. decreases c. the length of the bridge does not affect the corrosion processes d. only in soldering places e. excluding soldering sites | 1 | ТК-РК, ПА |
| 3. The number of removed hard tooth tissues along the chewing surface under the artificial crown depends on a. material for the manufacture of crowns b. fixing material c. age of the patient d. the anatomical origin of the tooth | 1 | ТК-РК, ПА |
| 4. Central occlusion is defined by signs. a. dental, articular, muscular b. lingual, muscular, articular c. facial, articular, muscular d. facial, articular, dental e. dental, swallowing, facial | 1 | ТК-РК, ПА |
| 5. Color is considered to be the reference color in dentistry. a. gray b. blue c. green d. white e. pink | 1 | ТК-РК, ПА |

*- указывается, при каком уровне контроля применяется оценочное средство (текущий контроль, в том числе рубежный контроль (ТК, ТК-РК), промежуточная аттестация (ПА), государственная итоговая аттестация (ГИА).

ПК-6: способностью к определению у пациентов основных патологических состояний, симптомов, синдромов стоматологических заболеваний, нозологических форм в соответствии с Международной статистической классификацией болезней и проблем, связанных со здоровьем, X просмотра

Раздел дисциплины: Съемное протезирование

Вид оценочного средства: Тестовые задания:

| Оценочное средство | Критерии оценки | Этап применения* |
| --- | --- | --- |
| 1. Unilateral distal unlimited (end) defect of the dentition (according to Kennedy’s classification) belongs to the class a. 1 b. 2 c. 3 d. 4 e. 5 | 1 | ТК-РК, ПА |
| 2. Classification of bridges by material: a. metallic, nonmetallic, combined b. removable, non-removable c. whole, compound d. tangents, flushing | 1 | ТК-РК, ПА |
| 3. Classification of bridges by design: a. whole, compound b. tangents, washing c. removable, non-removable d. city with a one-sided support, with a two-sided support | 1 | ТК-РК, ПА |
| 4. Classification of bridges according to the type of attachment: a. removable, non-removable b. metallic, nonmetallic, combined c. stamped-brazed, solid-cast, roasting, milling, polymerization d. tangents, flushing | 1 | ТК-РК, ПА |
| 5. Classification of bridges according to the method of manufacture: a. stamped-brazed, solid-cast, roasting, milling, polymerization b. tangents, washing c. with one-sided support, with two-sided support d. whole, compound | 1 | ТК-РК, ПА |
| 6. Classification of bridges according to the location of the abutment teeth: a. with one-sided support, with two-sided support b. stamped-brazed, solid-cast, roasting, milling, polymerization c. whole, compound d. dorsal, equatorial e. tangents, washings | 1 | ТК-РК, ПА |

*- указывается, при каком уровне контроля применяется оценочное средство (текущий контроль, в том числе рубежный контроль (ТК, ТК-РК), промежуточная аттестация (ПА), государственная итоговая аттестация (ГИА).

ПК-8:  Способность к определению тактики ведения больных с различными стоматологическими заболеваниями.

Раздел дисциплины: Несъемное протезирование

Вид оценочного средства: Тестовые задания:

| Оценочное средство | Критерии оценки | Этап применения* |
| --- | --- | --- |
| 1. Plastic crowns are applied - on the posterior teeth + on front teeth | 2 | ТК-РК, ПА |
| 2. Positive properties of plastic crowns: + simple manufacturing technology - wear resistance + low cost + relative aesthetics - high aesthetics - color tolerance | 1, 3, 4 | ТК-РК, ПА |
| 3. Contraindications to prosthetics with pin teeth + curvature of the root, inadequate filling of the canal, its small size, deep occurrence of the root under the gum + deep bite or deep overlap | 1,2 | ТК-РК, ПА |
| 4. Stump pin constructions are used - on single root teeth - on multiroot teeth + on all teeth | 3 | ТК-РК, ПА |
| 5. In the manufacture of pin construction the length of the pin from the length of the root is - 1/3 - 1/2 + 2/3 | 3 | ТК-РК, ПА |
| 6. Stump pin tabs are made on + incisors + canines + premolars | 1,2,3 | ТК-РК, ПА |
| 7. What type of metal crown with a pin, lined with plastic - according to Richmond - according to Kopeikin + according to Akhmedov - by Ilina - Markosyan | 3 | ТК-РК, ПА |
| 8. The manufacture of a complete anatomical crown is shown at: - IROPZ = 0,2 - IROPZ from 0.3 to 0.6 + IROPZ 0,6 -0,8 | 3 | ТК-РК, ПА |

*- указывается, при каком уровне контроля применяется оценочное средство (текущий контроль, в том числе рубежный контроль (ТК, ТК-РК), промежуточная аттестация (ПА), государственная итоговая аттестация (ГИА).

Раздел дисциплины: Съемное протезирование

Вид оценочного средства: Тестовые задания:

| Оценочное средство | Критерии оценки | Этап применения* |
| --- | --- | --- |
| 1. The indications for the manufacture of the bridge is a. included dentition defect b. defect of the tooth crown c. pathological abrasion d. severe periodontitis e. terminal unilateral defect of the dentition | 1 | ТК-РК, ПА |
| 2. A double-layer impression is obtained using impression materials. a. silicones b. alginate c. solid crystal d. thermoplastic e. hydrocolloid | 1 | ТК-РК, ПА |
| 3. The collapsible plaster model is molded during manufacture. a. solid crown b. stamped crown c. partial denture d. bridge e. plate repair prosthesis | 1 | ТК-РК, ПА |
| 4. Indications for the manufacture of composite bridge a. large convergence of teeth, limiting the defect b. mobility of abutment teeth c. end defect in the dentition d. the greater the length of the defect of the dentition | 1 | ТК-РК, ПА |
| 5. Causes of fuzzy display of the cervical area of the prepared tooth in a two-layer impression a. poorly performed retraction of the gums, insufficient drying of the tooth stump, insufficient compression when receiving the impression b. high fluidity of the corrective layer, too much corrective mass | 1 | ТК-РК, ПА |
| 6. Indications for the manufacture of the bridge a. included dentition defect b. lack of all teeth c. having all teeth d. one tooth e. defect in the coronal part of the tooth | 1 | ТК-РК, ПА |
| 7. Causes of the appearance of pores when imaging a prepared tooth upon receipt of a two-layer impression a. high fluidity of the corrective layer, insufficient amount of the corrective mass, insufficient drying of the tooth stump b. high hardness of the main (base) layer, excessive compression upon receipt of the impression | 1 | ТК-РК, ПА |
| 8. Possible complications when using metal-ceramic bridges a. increased erasure of natural antagonist teeth b. color change cladding c. rapid abrasion of the cladding d. increase chewing effectiveness e. improving the quality of life | 1 | ТК-РК, ПА |
| 9. Possible disadvantages of a stainless steel bridge. a. taste of metal in the mouth b. low chewing efficiency c. rapid abrasion of antagonist teeth d. the emergence of mobility of antagonist teeth e. bad breath | 1 | ТК-РК, ПА |
| 10. The cause of chipping of the ceramic facing of the bridge can be a. errors during firing and cooling of the ceramic cladding b. pronounced taper of the tooth stump c. poor oral hygiene d. the presence of a removable denture in the oral cavity e. the presence of artificial crowns on the antagonist teeth | 1 | ТК-РК, ПА |

*- указывается, при каком уровне контроля применяется оценочное средство (текущий контроль, в том числе рубежный контроль (ТК, ТК-РК), промежуточная аттестация (ПА), государственная итоговая аттестация (ГИА).

ПК-9:  Готовность к ведению и лечению пациентов со стоматологическими заболеваниями в амбулаторных условиях и условиях дневного стационара.

Раздел дисциплины: Несъемное протезирование

Вид оценочного средства: Тестовые задания:

| Оценочное средство | Критерии оценки | Этап применения* |
| --- | --- | --- |
| 1. The remains of cement on the surface of the teeth and dentures are removed with the help of: - excavator + probe - tweezers - trowels | 2 | ТК-РК, ПА |
| 2. What is the thickness of the solid crown from KHS? - 0.25-0.3 - 1.0-1.2 + 0.3-0.5 - 1.2-1.5 | 3 | ТК-РК, ПА |
| 3. What is the thickness of the metal-ceramic crown - 0.25-0.3 - 1.0-1.4 - 0.3-0.35 + 1.2-1.5 | 4 | ТК-РК, ПА |
| 4. What is the thickness of the plastic crown - 1.0-1.2 - 1.2-1.5 + 0.6-1.0 - 0.25-0.3 | 3 | ТК-РК, ПА |
| 5. For permanent fixation of fixed prostheses we apply: - repin + cements - oil dentin - water dentin - acrylic plastics | 2 | ТК-РК, ПА |
| 6. For making of working impressions in the manufacture of tabs we use materials: - solid crystal - alginate + silicone - zenkoxyeugenolovye | 3 | ТК-РК, ПА |
| 7. For double impressions _____ masses are used: - solid crystal + silicone - alginate - thermoplastic - zenkoxyeugenolovye | 2 | ТК-РК, ПА |
| 8. Alginate mass is used to obtain working impressions in the manufacture of ___ crowns: - cast - porcelain - metal-ceramic - metal-plastic + stamped | 5 | ТК-РК, ПА |
| 9. In the manufacture of metal-ceramic crowns working impression is done by: - alginate mass + silicone mass - any impression material with gum retraction - plaster - thermoplastic mass | 2 | ТК-РК, ПА |
| 10. In the manufacture of solid crown for the working impression we use: - alginate + silicone - zincoxyeugenol - thermoplastic - solid crystal | 2 | ТК-РК, ПА |
| 11. For permanent fixation of fixed prostheses we apply: - repin - oil dentin + Fuji - tempbond - water dentin | 3 | ТК-РК, ПА |
| 12. The plastic crown with respect to the gingival margin should be located: - Do not reach the gum by 0.5 mm - Do not reach the gum by 1.0 mm + at gum level - under the gum on 0.5 mm - under the gum by 1.0 mm | 3 | ТК-РК, ПА |
| 13. Is it possible to use porcelain crowns with the pathological abrasion of teeth? -not -possibly + possible, but after appropriate preparation - possible only when I degree - possible only when I and II degree | 3 | ТК-РК, ПА |

*- указывается, при каком уровне контроля применяется оценочное средство (текущий контроль, в том числе рубежный контроль (ТК, ТК-РК), промежуточная аттестация (ПА), государственная итоговая аттестация (ГИА).

Раздел дисциплины: Съемное протезирование

Вид оценочного средства: Тестовые задания:

| Оценочное средство | Эталон ответа | Уровень применения* |
| --- | --- | --- |
| 1. The intermediate part of the bridge in the region of the posterior teeth in relation to the gum a. does not concern b. adjoins to it on all surface c. adjacent only on the slopes of the alveolar ridge d. touches on top of the alveolar ridge at two points e. touches on top of the alveolar ridge at one point | 1 | ТК-РК, ПА |
| 2. All the side walls of the abutment teeth in the manufacture of the solder bridge are prepared a. parallel with each other b. with a tilt in the direction of the defect of the dentition c. inclined away from the dentition defect d. in parallel with the adjacent tooth e. only parallel to the longitudinal axis of the tooth | 1 | ТК-РК, ПА |
| 3. Modeling of the body of metal-ceramic bridge is carried out a. at one time with the modeling of the crown b. before modeling crowns c. at the stage of fitting the crown on the model d. after the stage of fitting crowns in the clinic e. after the laboratory stage of the manufacture of support crowns | 1 | ТК-РК, ПА |
| 4. The shape of the intermediate part of the bridge in the region of the posterior teeth in relation to the gum a. washable b. tangent c. saddle d. can be any e. depends on the length of the dentition defect | 1 | ТК-РК, ПА |
| 5. For soldering stainless steel crowns, solder is used on the basis of a. silver b. gold c. platinum d. titanium e. nickel | 1 | ТК-РК, ПА |
| 6. During the fitting of cast bridges, the accuracy of the fit of the crowns to the stumps of the abutment teeth is assessed using a. elastic impression materials b. base wax c. gypsum d. city copy paper e. water dentin | 1 | ТК-РК, ПА |
| 7. Fixed bridges according to the method of transferring chewing pressure are (according to Rumpel's classification) to a. physiological b. semi-physiological c. non-physiological d. combined e. leaning | 1 | ТК-РК, ПА |
| 8. The shape of the intermediate part of the bridge in the region of the posterior teeth in relation to the gum a. washable b. tangent c. saddle d. can be any e. depends on the length of the dentition defect | 1 | ТК-РК, ПА |
| 9. The shape of the intermediate part of the bridge in the area of the front teeth a. tangent b. saddle c. wash d. diatorial e. combined | 1 | ТК-РК, ПА |
| 10. At the stage of fitting a solid bridge prosthesis, the doctor receives the prosthesis from the dental laboratory on a. folding plaster model b. wax plate c. metal stamps d. plaster stamps e. wax basis | 1 | ТК-РК, ПА |
| 11. Bleaching of the non-removable stainless steel bridge after soldering is carried out in a. mixtures of acids with the addition of alkali b. concentrated alkalis c. concentrated acids d. alkaline mixtures with added water e. in any of the listed liquids | 1 | ТК-РК, ПА |
| 12. Separation - the stage of preparing the teeth for artificial crowns, which includes the preparation of hard tooth tissues with a. contact surfaces b. vestibular surface c. oral surface d. occlusal surface e. creation of a scarp | 1 | ТК-РК, ПА |
| 13. The wax used to define the central occlusion is called a. basic b. sticky c. profile (waxed) d. byugelny e. modeling | 1 | ТК-РК, ПА |
| 14. Abrasive odontopreparing tools are a. diamond burs b. carbide cutters c. carbide burs d. steel burs e. polishing brushes | 1 | ТК-РК, ПА |
| 15. The intermediate part of the bridge can be represented a. facet b. implant c. veneer d. tab e. crown | 1 | ТК-РК, ПА |
| 16. The supporting parts of the bridge are a. crowns/tabs b. artificial teeth, half-crowns c. pin teeth d. artificial teeth | 1 | ТК-РК, ПА |
| 17. The intermediate part of the bridge can be represented a. facet b. semi-funnel c. crown d. veneer e. tab | 1 | ТК-РК, ПА |
| 18. Alginate impression mass is mixed on a. cold water b. hot water c. saline solution g. 3% solution of salt e. attached to the material catalyst | 1 | ТК-РК, ПА |
| 19. According to the method of manufacture distinguish bridges. a. solid cast b. fixing c. splinting d. city faceted e. recovery | 1 | ТК-РК, ПА |
| 20. Gypsum model for alginate imprint material must be cast no later than a. 15 minutes b. 60 minutes c. 2 minutes d. 48 hours e. 24 hours | 1 | ТК-РК, ПА |
| 21. In the manufacture of cantilever fixed prosthesis is negative a. the presence of tilting moment in the area of the abutment teeth b. need for depulpation of abutment teeth c. preparation of a large number of teeth d. poor aesthetic quality e. grinding a large amount of tissue of the abutment teeth | 1 | ТК-РК, ПА |
| 22. The correct sequence of preparation of the framework of the metal-ceramic crown before applying and firing the ceramic mass a. sandblasting, degreasing the frame, creating an oxide film (firing) b. creation of oxide film (firing), degreasing the frame, sandblasting c. degreasing of the frame, sandblasting, the creation of an oxide film (firing) d. skimming of the frame, the creation of an oxide film (firing), sandblasting e. the creation of an oxide film (firing), degreasing the frame, sandblasting | 1 | ТК-РК, ПА |
| 23. The difference in height of the lower part of the face in a state of relative physiological rest and at the closure of the dentition in the position of the central occlusion is on average (in mm): a. 0.5-1.0 b. 2-4 c. 5-6 d. 7-8 e. 9-10 | 1 | ТК-РК, ПА |
| 24. Materials for permanent fixation. a. visfat-cement b. water dentin c. repin d. dentine paste | 1 | ТК-РК, ПА |

*- указывается, при каком уровне контроля применяется оценочное средство (текущий контроль, в том числе рубежный контроль (ТК, ТК-РК), промежуточная аттестация (ПА), государственная итоговая аттестация (ГИА).